# Lab 3: Routing Protocols – RIP and OSPF

# Demo Question 1 (RIP):

Configure 2 Routers, 8 PCs into 4 subnetworks and simulate RIP protocol for routing. Check connectivity of all.

Steps:

- Select 2 routers. One 1841 and one 2811.

- For each router do the following:

  - Click on router

  - Switch it off

  - Drag and drop WIC-2T to slot0

  - Switch on

- Connect 4 2950T switches on the routers, with 2 PCs each

- Connect the Switches, PCs and Router with “Straight through” cable

- Connect the two routers using “Serial DTE” cable

- Click on Router0, and do the following configurations:

  - Click on router

  - Click on CLI

  - Run these commands

    - enable

    - config t

    - interface serial 0/0/0

    - ip address 10.10.10.1 255.255.255.252

    - no shutdown

    - exit

    - interface fastEthernet 0/0

    - ip address 192.168.10.1 255.255.255.0

    - no shutdown

    - exit

    - interface fastEthernet 0/1

    - ip address 192.168.20.1 255.255.255.0

    - no shutdown

    - exit

    - router rip

    - network 192.168.10.0

    - network 192.168.20.0

    - network 10.10.10.0

    - exit

- Do the same for the Router1, with different IP addresses

- Click on simulation -> Add fliters -> Remove all except ICMP and RIP

- Add PDU packets

- Run simulation

# Lab exercise Quesiton 1:

Configure 4 Routers and 2 PCs for RIP routing protocol. Show connectivity between all.

Steps:

Select 4 routers. One 1841 and one 2811, and two 2620XM.

- For each router do the following:

  - Click on router

  - Switch it off

  - Drag and drop WIC-2T to slot0

  - Switch on

- Connect the PCs and Router with “Copper cross over” cable

- Connect the four routers using “Serial DTE” cable

- Click on Router0, and do the following configurations:

  - Click on router

  - Click on CLI

  - Run these commands

    - enable

    - config t

    - interface serial 0/0/0

    - ip address 10.10.10.1 255.255.255.252

    - no shutdown

    - exit

    - interface fastEthernet 0/0

    - ip address 192.168.10.1 255.255.255.0

    - no shutdown

    - exit

    - router rip

    - network 192.168.10.0

    - network 10.10.10.0

    - exit

- Do the same for all the other routers, with different IP addresses as per their interface.

- Click on simulation -> Add fliters -> Remove all except ICMP and RIP

- Add PDU packets

- Run simulation

# Demo Question 1 (OSPF)

Configure 2 subnetworks, with 3 routers, 4 PCs (2 each), using OSPF routing protocols. Show connectivity between all.

Steps:

Select 4 routers – 2621XM.

- For each router do the following:

  - Click on router

  - Switch it off

  - Drag and drop WIC-2T to slot0

  - Switch on

- Connect the PCs and Router with “Straight Through” cable to the switches and them to routers

- Connect the four routers using “Serial DTE” cable

- Click on Router, and do the following configurations:

  - Click on router

  - Click on CLI

  - Run these commands

    - enable

    - config t

    - interface serial 0/0/0

    - ip address 10.10.10.1 255.255.255.252

    - no shutdown

    - exit

    - interface fastEthernet 0/0

    - ip address 192.168.10.1 255.255.255.0

    - no shutdown

    - exit

    - router ospf 1

    - network 192.168.10.0 0.0.0.255 area 0

    - network 10.10.10.0 0.0.0.3 area 0

    - exit

- Do the same for all the other routers, with different IP addresses as per their interface.

- Click on simulation -> Add fliters -> Remove all except ICMP and RIP

- Add PDU packets

- Run simulation

# Demo Question 1 (DHCP):

Configure two routers and 4 subnetworks using 13 PCs. Use one of the routers as DHCP server and the other as a relay agent. Show DHCP IP assignment for the network on server router and the network on relay router.

Steps:

Select 2 routers – 2811.

- For each router do the following:

  - Click on router

  - Switch it off

  - Drag and drop WIC-2T to slot0

  - Switch on

- Connect the PCs and Router with “Straight Through” cable to the switches and them to routers

- Connect the four routers using “Serial DTE” cable

- Click on Router0 and do the following configurations:

  - Click on router

  - CLI

  - Type the following

    - enable

    - config t

    - interface serial 0/0/0

    - ip address 10.10.10.2 255.255.255.252

    - no shutdown

    - exit

    - interface fastEthernet 0/0

    - ip address 192.168.10.1 255.255.255.0

    - no shutdown

    - exit

    - ip dhcp excluded-address 192.168.10.1 192.168.10.20

    - ip dhcp pool MyPool1

    - network 192.168.10.0 255.255.255.0

    - default-router 192.168.10.1

    - dns-server 192.168.10.100

    - exit

    - ip dhcp pool MyPool2

    - network 192.168.30.0 255.255.255.0

    - default-router 192.168.30.1

    - dns-server 192.168.10.100

    - exit

- For the second Router1, do the following:

  - Click on router

  - CLI

  - Configure the interface IP addresses

  - Add the following code:

    - interface fastEthernet 0/0

    - ip address 192.168.30.1 255.255.255.0

    - ip helper-address 10.10.10.2

    - exit

    - router rip

    - network 10.10.10.0

    - network 192.168.30.0

    - exit

- Click on any PC -> Desktop -> IP config -> Change IPv4 settings to DHCP